DogTracker Project Guide

DogTracker is a React Native Android app that connects to the Heltec V4 DogGPS-Master3 over BLE. It receives the OLED dataset generated by the Master and displays GPS, distance, activity, signal and battery values.

Current Features

- Scan and connect to DogGPS-Master3.

- Subscribe to BLE GATT notifications.

- Display Slave and Master GPS, distance, speed, satellites, HDOP, activity, RSSI, SNR and battery status.

- Support BLE permissions required by Android 8.1 and Android 12+.

- Build a standalone Release APK that does not require USB or Metro.

Module Ownership

text

App.js                    # App entry point and BLE realtime UI

src/

├─ ble/                    # Task A: BLE scan, connection and parsing

│  ├─ BleService.js

│  ├─ BleScanner.js

│  └─ BleParser.js

├─ gps/                    # Task B: phone GPS interface

│  └─ LocationService.js

├─ database/               # Task C: SQLite interface

│  └─ DogDatabase.js

├─ screens/                # Task D: screen components

│  ├─ MapScreen.js

│  └─ HistoryScreen.js

├─ map/                    # Task E: map interface

│  └─ DogMap.js

└─ models/                 # shared data format

└─ DogStatus.js

Each Task should modify its own directory. Cross-task data must pass through src/models/DogStatus.js or an explicit module interface. Avoid putting feature logic back into App.js.

| Task | Owner | Responsibility |
| --- | --- | --- |
| A: BLE | src/ble/ | Permissions, scan, GATT connection, Notify, Base64 and JSON parsing |
| B: GPS | src/gps/ | Phone location permission and location provider |
| C: SQLite | src/database/ | Schema, status history persistence and queries |
| D: UI | src/screens/ | Screens and navigation |
| E: Map | src/map/ | Map component and coordinate presentation |

Shared DogStatus Model

BLE data is normalized by toDogStatus() in src/models/DogStatus.js. UI, GPS and database modules should use the normalized camelCase fields, not raw BLE keys.

text

slaveLat, slaveLon, masterLat, masterLon

distanceMeters, speedKmh, satellites, hdop

activity, activityValid, rssi, snr

batteryMillivolts, batteryPercentage, batteryValid

masterBatteryMillivolts, masterBatteryPercentage, masterBatteryValid

gpsTime, activityTime, type, sequence, length

BLE Protocol

| Item | Value |
| --- | --- |
| Device name | DogGPS-Master3 |
| Service UUID | 7f510001-6d9e-4e2f-a671-8f3f2d49a001 |
| Data Characteristic UUID | 7f510002-6d9e-4e2f-a671-8f3f2d49a001 |
| Direction | Master Notify to phone |

The Master sends a compact OLED dataset. Current compact keys include:

text

slat/slon   Slave coordinates

mlat/mlon   Master coordinates

dst         distance in meters

spd         speed in km/h

sat/hd      satellites / HDOP

act/av      activity / activity-valid flag

bmv/bp/bv   Slave battery millivolts / percentage / valid flag

mbmv/mbp/mbv Master battery millivolts / percentage / valid flag

rssi/snr    LoRa signal quality

gt/at       GPS and activity timestamps

The BLE Characteristic must have one publisher. The current Master publishes the complete OLED dataset from its OLED task to prevent competing long and short JSON notifications.

Development

Requirements:

- Windows

- Node.js >= 22.11.0

- Android SDK, Platform Tools and JDK

- React Native 0.87.0

Install dependencies:

powershell

npm install

Start Metro and run a USB-connected Debug build:

powershell

npm start

adb devices

adb reverse tcp:8081 tcp:8081

npm run android

Debug builds require Metro. Do not use npm run android when the goal is a USB-independent installation because it installs a Debug APK.

Test and Release

powershell

npm run lint

npm test -- --runInBand

Build a standalone APK:

powershell

cd android

.\gradlew assembleRelease

cd ..

Output:

text

android\app\build\outputs\apk\release\app-release.apk

Install it on a connected device:

powershell

adb install -r android\app\build\outputs\apk\release\app-release.apk

The Release APK includes the JavaScript bundle and can start without USB or Metro. BLE still requires Bluetooth and the Android location/Bluetooth permissions requested by the app.

Git Collaboration

Use one feature branch per Task:

powershell

git switch -c feature/task-name

git status

git add <your-files>

git commit -m "feat: describe the change"

git push -u origin feature/task-name

Open a Pull Request into main. Before merging, run lint and tests; BLE or Android permission changes should also pass a Release build.

Reserved Modules

The following modules are intentionally interfaces/placeholders for later Tasks:

- src/gps/LocationService.js: phone GPS provider

- src/database/DogDatabase.js: SQLite driver and persistence

- src/map/DogMap.js: map implementation

- src/screens/MapScreen.js: map screen

- src/screens/HistoryScreen.js: history screen